NOTE:  This document is now rather out of date.  The most recent

information can be found on the World Wide Web at

http://www.ultranet.com/~sstv

This is constantly being updated with new product announcements,

operating tips, convention reports, and all the latest news.

=======================================================================

SSTV Information

----------------

Slow Scan Television has been around for a few decades but it never

got very much attention.  The reason is that commercial equipment was

very expensive and and it was much too complicated for most people to

homebrew.  The situation has changed radically in the last couple years.

Rather than using expensive special purpose hardware, most of the newer

systems are using personal computers to do most of the work.  There is

now a wide assortment of free software that uses very simple interfaces

and ready-to-use commercial systems at affordable prices. New people are

showing up on SSTV everyday.

Although SSTV activity has exploded during the last couple years, it's

still hard to find much modern information.  The most recent SSTV

handbook published in the U.S. (that I know of) is now almost 20 years old.

Technology has changed quite a bit since then.

A very popular introductory book about Ham Radio (no names mentioned

but it's available at Radio Shack) devotes only a few sentences to SSTV

and describes it as 8 second black & white pictures.  It's been nearly

all color pictures for many years.

The CQ 1994 Equipment Buyer's Guide lists only two SSTV products and

neither is manufactured anymore.

Here is one of my attempts to help combat this lack of information.

This file contains some background on SSTV, a list of equipment available,

and pointers to other sources of information.

Detailed specifications and prices have been avoided because that could

be considered advertising.  This file is being distributed over the

Internet and Ham Radio Packet BBSs where advertising is not appropriate.

Some outdated and just plain wrong information keeps getting passed

back and forth between various lists of SSTV equipment.  About a year

ago I sent letters to 29 different suppliers asking about their SSTV and

related products.  Some letters were returned as undeliverable.  Most

of those who did not respond have been dropped from the list.   A couple

that didn't respond are still listed because the products are believed

to be still available.

As with anything else, there are some good products available and some

real junk being sold with deceptive advertising.  Just get on the air

and ask what people are using and how they like it.  Caveat emptor!

Please send additions and corrections to johnl@world.std.com and I'll

distribute a new version occasionally.

Let me know where you saw this file; I'd be interested in knowing how

far it gets propagated.

Version of January 1995.

------------------------

Background

----------

There are several different ways to send images over ham radio:

1. RTTY art - Remember when people made pictures from characters?

2. FSTV (Fast Scan TV - Also called ATV) - Similar to broadcast TV.

Full motion, color, sound, etc.  Restricted to UHF and up because

a signal requires several MHz of bandwidth.

3. WEFAX (weather facsimile) - Very high resolution gray scale images

sent by audio tones over a period of minutes.

4. SSTV (Slow Scan TV) - Low to medium resolution still images sent

through audio channels over a period of several seconds to a few

minutes.  Mostly color these days.

5. Digital SSTV - We have the technology but I haven't seen any proposals

yet.

Transmission Modes

------------------

The original 8 second SSTV transmission mode (around 1958) had the following

characteristics:

black = 1500 Hz

white = 2300 Hz

gray levels in between

120 scan lines

15 lines per second

5 mS of 1200 Hz for horizontal sync

30 mS of 1200 Hz for vertical sync

Since that time many more modes have been invented, sometimes to add new

capabilities, sometimes the result of Not Invented Here syndrome.

The major groups are:

Robot - Introduced with the Robot scan converters (California).

Wraase - Introduced with the Wraase scan converters (Germany).

Martin - Developed by Martin Emmerson (England).

First available as replacement PROMs for Robot 1200C.

Scottie - Developed by Eddie Murphy (Scotland).

First available as replacement PROMs for Robot 1200C.

AVT - Developed by Ben Blish/Williams (Montana).

First available in the AVT system.

The Robot, Wraase, Martin, and Scottie modes are all closely related.

They all use the tones above for black, white, and gray levels.

They all have 1200 Hz horizontal sync, although some Martin and Scottie

implementations rely on accurate crystal oscillators and ignore the

horizontal sync once synchronized.  Color is generally transmitted by sending

each scan line 3 times, once each for red, green, and blue components.

Robot is different from the rest in that it encodes colors with

luminance (Y) and chrominance (R-Y and B-Y) instead of R,G,B.

Each of these modes has a few different speeds, usually providing 120 or 240

scan lines and varying degrees of horizontal resolution.  (Actually some send

128 or 256 lines but the top 8 or 16 are always a fixed gray scale,

leaving 120 or 240 usable lines.)   In general, the trade-off is time vs.

image quality: the modes with the best image quality require the most time.

The Robot modes have a much longer vertical sync (called VIS) containing

7 bits of information and a parity bit.  This identifies the format of the

following image so manual selection is not required on systems that

recognize it.  Everyone else has adopted the Robot VIS coding and assigned

themselves unused codes in the original specification.  Unfortunately,

Scottie DX and one of the AVT modes use the same VIS code due to a

lack of cooperation among developers.

The AVT mode is radically different from the rest.  It has no horizontal

sync at all; very accurate crystal oscillators are required to prevent

slanted pictures.  After 3 repetitions of the usual Robot VIS code,

it has a digital header with 32 repetitions of the transmission mode, and

a sequence number.  It is only necessary to receive one of the 32 groups

correctly to achieve synchronization.  For more details see the AVT article

in CQ-TV mentioned in the bibliography.

Frequencies

-----------

By convention, SSTV operation is generally found on only a few frequencies.

3.845

3.857

7.171

14.230

14.233

21.340

28.680

144.5

Nets

----

Two different Slow Scan nets meet on Saturdays at 15:00 and 18:00 UTC

on 14.230 MHz.

Scan Converters

---------------

A scan converter is a device that converts images in one TV standard to

another.  In this case we interested in converting between SSTV (for

transmission with voice radios) and NTSC/PAL so we can use Camcorders,

TV monitors, etc. for creation and display of images.

A couple years ago anyone who was serious about SSTV had a Robot 1200C.

It is a complete system dedicated to SSTV.  Just connect it to

- Color TV camera (either NTSC or PAL).

- Color TV set or monitor.

- Speaker and Mic connectors of a transceiver.

- Tape recorder for picture storage.

It has 4 black & white modes (one compatible with original 8 second) and

4 color modes with different transmission times and resolutions.

It displays images with 256 x 240 resolution with 18 bits per pixel.

That's more than 250,000 colors.

Robot Research

5636 Ruffin Road

San Diego, CA 61927

It also has a parallel port for connection to a home computer.  Several

different programs for transferring images to/from the 1200C and for

performing various other functions are available.

ART (Amiga Robot Terminal - for Amiga computer)

Tom Hibben KB9MC

Mule Hollow Road

Box 188

DeSoto, WI 54624

(608) 648 2033

GEST

Torontel

174 Bellamy Rd. N.

Scarborough, Ontario

Canada M1J 2L5

(905) 888 9054

or

Royal Electronics (Canada)

336 Goodram Drive

Burlington, Ontario

Canada L7L 2K1

(905) 637 9622

Hi-Res (for IBM PC)

Tom Jenkins N9AMR

5968 S. Keystone Ave.

Indianapolis, IN 46227

(317) 784 6118

Robot Helper

William Montgomery VE3EC

482 Townsend Ave.

Burlington, Ontario

Canada L7T 2B3

(416) 681 0509

SCAN

Bert Beyt W5ZR

301 Tampico St.

New Iberia, LA 70560

(318) 365 5263

Replacement PROMs are available to give the 1200C other transmission modes

such as Wraase, Martin, Scottie, and AVT.  PROMs available from:

Martin Emmerson G3OQD

6 Mount Hurst Rd.

Hayes, Bromley

Kent BR2 7QN

England

See "SSTV with the Robot 1200C Scan Converter and the Martin Emmerson

EPROM Version 4.0" in the July 1991 issue of 73 Amateur Radio Today for

more details.

The 1200C was discontinued late 1992.

A 1200C equivalent is available:

DFM 1200 USA Scan Converter

Felipe Rojas AB4QC

3475 Shady Woods Circle

Lawrenceville, GA 30244

or

Donald Lucarell K8SQL

265 Outlook

Youngstown, OH 44504

or

Muneki Yamafuzi JF3GOH

2-3-2, Ohyodonaka Kitaku

Osaka 531

Japan

Semi-kits containing 3 PC boards and the hard to find

components.

The SUPERSCAN 2001 is similar to the Robot 1200C but a more modern

design with several improvements.  It can hold four 256 x 240 images with

about 250,000 colors.  New features include mouse support, titling

without a computer, both parallel and serial interfaces, and a CMOS RAM

with battery backup to save parameters.

Now available are:

* Semi-kit with 3 printed circuit boards, an EPROM developed

by Martin Emmerson, and instructions.

* High stability crystal oscillator.

* Interface card for IBM PC.

* Packages of hard-to-find components.

The builder must gather up all the other components which go on the boards.

Jad Bashour

55 Hampton Road

London N15 3SX

England

Tel. 081 889 0486

The Tasco TSC-100A Scan Converter was displayed at Dayton '94.  It is an

amazingly compact (5.5 x 1.6 x 8.5 inches) DSP-based device.  It operates

from 12 volts making it ideal for portable/mobile use.

Video input/output:     NTSC.  Future PAL version planned.

Frame memory:           1 standard, 3 more optional.

Serial port:            For mouse or connection to computer.

Modes:                  Robot 36 & 72, AVT 90 & 94.

U.S. distributor:

EDCO Electronic Distributors Co.

325 Mill Street

Vienna, VA 22180

(703) 938 8105

Digital Signal Processing

-------------------------

Digital Signal Processing (DSP) could be the next major breakthrough in

improving SSTV reception while maintaining compatibility with existing

equipment.  DSP uses an Analog to Digital (A/D) converter to sample the

audio signal.  After that it's all software.

DSP can simulate analog circuitry or do types of processing not even

possible with the analog approach.  DSP isn't magic and it doesn't

guarantee superior performance.   It all depends on the skill of the

person writing the DSP software.

Home Computers

--------------

Now that most home computers have plenty of memory and excellent color

graphics, the most cost effective method is to use a computer with a

suitable interface and software.

Note that if you want to send a picture of anything real, you will also

need a frame grabber which will add a few hundred more $ to the total

system cost.

Amiga

-----

The AVT Master system was once very popular during for both SSTV and WEFAX.

It is composed of an interface that attaches to the parallel port and

software.

It has images with up to 4096 colors, all the popular transmission modes,

builtin graphics editor, text generation, image processing techniques to

clean up noisy pictures, and loads of other features.

It was discontinued Fall 1993 when AEA dropped all of its Amateur

Television products.

IBM PC

------

When I first typed up this file, there was only a very crude B&W

system available.

The situation has changed dramatically in the last year or two.  There are

now several PC-based color SSTV systems available.

Pasokon TV

Absolute Value Systems

115 Stedman St. # B

Chelmsford, MA 01824-1823

(508) 256 6907

See QST, January 1993 for details.

PC SSTV 5.0

Software Systems Consulting

150 Avenida Cabrillo, "C"

San Clemente, CA 92672

SSTV Explorer  (Receive-only)

Radioware Corp.

P.O. Box 1478

Westford, MA 01886

(508) 452 5555

(800) 950 9273

Product review in QST, April 1994.

ViewPort VGA (developed by KA2PYJ)

A & A Engineering

2521 West LaPalma, Unit K

Anaheim, CA 92801

Construction article in 73, August 1992.

All the systems have some kind of hardware interface (modem or just

demodulator) and software that runs on the PC.  All require a VGA display.

But there the similarities end.

Interestingly, much different approaches were taken with the hardware.

Pasokon TV uses an interface that fits into an expansion slot inside the

computer.   ViewPort VGA uses an external interface that connects to the

printer port.   PC SSTV and SSTV Explorer use small interfaces that

plug into a serial port.

Software varies quite a bit in terms of SSTV modes implemented, support

for super VGA cards with more than 256 simultaneous colors, ease of use, and

other features.

HiRes 32 is a new version of the popular HiRes program.  It is a paint

program designed specifically for use with the PC-based SSTV systems.

It has fancy text and a wide variety of special effects.

HiRes 32

Tom Jenkins N9AMR

5968 S. Keystone Ave.

Indianapolis, IN 46227

(317) 784 6118

Most of the recent growth of SSTV activity has been from the use of low

cost interfaces and software on personal computers rather than expensive

dedicated hardware.  The next logical step is to use other common hardware

which wasn't even intended for to be used for this purpose.

SLOW SCAN TV for the SOUND BLASTER

Gene Harlan WB9MMM

Harlan Technologies

5931 Alma Drive

Rockford, IL 61108

(815) 398 2683

As the name implies, it uses the popular Sound Blaster

sound card from Creative Labs.

A demo version is available through Compuserve:

GO HAMNET

Library 6

SLOWSCAN.ZIP

Multimode Systems

-----------------

The MFJ-1278B, primarily intended for packet radio, RTTY, AMTOR, Morse

code, etc. is also capable of SSTV with the optional MultiCom software.

MFJ Enterprises, Inc.

P.O. Box 494

Mississippi State, MS 39762

(601) 323 5869

The AEA DSP-2232 has DSP modem software for SSTV.  I talked to someone

in sales there a while back.  They have no plans to complete the

other half: the PC software to make the modem useful.

The BMK-MULTY is software for sending and receiving AMTOR, RTTY, WEFAX,

SSTV, etc.

Schnedler Systems, AC4IW

25 Eastwood Rd.

P.O. Box 5964

Asheville, NC 28813

(704) 274 4646

Mac

---

Many people have asked many times and there doesn't seem to be

anything available.

Others

------

There have been SSTV implementations for other machines such as the Radio

Shack COCO and the Atari computers.  (See Bibliography, below.)  CQ-TV also

mentions SSTV for machines we never heard of in the USA.

Free Software

-------------

There are several free SSTV programs available that will receive, and

perhaps transmit, with very simple interfaces.  Look around the ARRL BBS

(phone (203) 666 0578 or e-mail to info@arrl.org), Internet archives

(ftp to world.std.com, ftp.cs.buffalo.edu, oak.oakland.edu or other

SimTel mirrors), or the ham radio sections of other on-line services

or BBSs.

Look for file names resembling:

EZSSTV.ZIP

JVFAX??.ZIP

SLOWSCAN.ZIP

SSTVFAX2.ZIP

VESTER?.ZIP

Naturally, these don't offer all the features of some of the commercial

systems but they are a wonderful way to get a taste of SSTV with little

or no cost.

Publications

------------

A completely new up to date SSTV handbook is available on the World

Wide Web.  It contains sections on history, typical images, how

to get started with little or no cost, commercial systems, homebrew

projects, bibliography, glossary and more.  Its Universal Resource

Locator is

http://www.ultranet.com/~sstv

There are only two hardcopy handbooks printed in recent years:

SSTV Handbook, 1st Edition 1994

editor JA6OAC

ISBN 4-7898-1011-9

(In Japanese so most of it makes no sense to me but

many of the illustrations are interesting.)

Slow Scan Television Explained

by Mike Wooding G6IQM, 1992

ISBN 0-9513779-3-0

(available from BATC)

The only other SSTV handbook, that I know about, was written by Don Miller

W9NTP and Ralph Taggart WB8DQT about 20 years ago.

The SSTV section of the 1995 ARRL Handbook is completely rewritten

and greatly expanded from earlier versions.  See pages 12.37 to 12.46.

There is a good SSTV overview in the January 93 issue of QST.

There are a few magazines that specialize in Amateur Television.  These are

mostly oriented toward fast scan TV but SSTV and WEFAX are mentioned

occasionally.

The SPEC-COM Journal

P.O. Box 1002

Dubuque, IA 52004-1002

CQ-TV

British Amateur Television Club

Dave Lawton G0ANO

Greenhurst, Pinewood Road

High Wycombe, Bucks HP12 4DD

England

Amateur Television Quarterly

1545 Lee St.

Suite 73

Des Plaines, IL 60018

Amateur TV Today!

QCD Publications

1492 Highway 30

Tipton, IA 52216-9335

Booklets of old ATVQ and A5 articles are available from:

ESF Copy Service

4011 Clearview Dr.

Cedar Falls, IA 50613

A newsletter devoted to SSTV:

VISION

International Visual Communications Association (IVCA)

P.O. Box 140336

Nashville, TN 37214

Bibliography

------------

I've purposely excluded the scores of articles describing modifications

to the Robot 400 (an early Black & White only system) because they're all

in the booklets from ESF and of little interest to someone without a

Robot 400.

Abrams, Clay K6AEP & Taggart, Ralph WB8DQT, "Color Computer SSTV", 73,

Nov-Dec 84.

Cameroni, Giuseppe I2CAB & Morellato, Giancarla I2AED (translated by

Jim Grubbs K9EI), "get on SSTV - with the C-64", ham radio, Oct. 86, P. 43.

Churchfield, Terry K3HKR, "Amiga AVT System", 73 Amateur Radio, Jul 89, p. 29.

Goodman, Dick WA3USG, "SSTV with the Robot 1200C Scan Converter and the

Martin Emmerson EPROM Version 4.0", 73 Amateur Radio Today, Jul 91, p. 46.

Harlan, Gene WB9MMM, "Slow Scan TV with the Sound Blaster",

QEX, May 93.

Langner, John WB2OSZ, "Color SSTV for the Atari ST", 73 Amateur Radio,

Dec 89, p. 38, Jan 90, p. 41.

Langner, John WB2OSZ, "SSTV - The AVT System Secrets Revealed", CQ-TV 149

(Feb 90), p. 79.

Langner, John WB2OSZ, "Slow Scan Television - It isn't expensive anymore",

QST, Jan. 93, p. 20.

Langner, John WB2OSZ, "Slow-Scan TV Questions and Answers", Radio Fun,

Feb. 95, p. 13.

Montalbano, John KA2PYJ, "The ViewPort VGA Color SSTV System", 73, Aug 92,

p. 8.

Pagel, Paul N1FB, "Radioware SSTV Explorer" (product review), QST, Apr 94,

p. 80.

Schick, Martin K. KA4IWG, "Color SSTV and the Atari Computer", QST, Aug 85.

Taggart, Dr. Ralph E. WB8DQT, "The Romscanner", QST, Mar. 86, p. 21.

Vester, Ben K3BC, "An Inexpensive SSTV System", QST, Jan. 94, p. 27.

Vester, Ben K3BC, "An Inexpensive SSTV System continues to Grow",

QST, Dec. 94, p. 22.

Conclusion

----------

Most hams are scared away from trying SSTV because they still think it

HAS TO be expensive or difficult.  That WAS true but it's not anymore.

SSTV activity has exploded in the last year with the introduction of

low cost IBM PC-based SSTV systems.  People are constantly telling me

that using SSTV is the most fun they've ever had with ham radio.